Review

- Midterm all up to include MOSFET

- 5-6 problems

- No sheet but provide all constants

- Bring calculator no phone

Lecture 1

Lecture 2

Intrinsic semiconductor 指的是纯净半导体，没有人为掺杂 donor 或 acceptor。

与之相对的是：extrinsic semiconductor

对温度极其敏感。室温下硅常用 1010 cm-3，但换温度就不能随便用了。

电子只能出现在导带或者价带，不能出现在禁带区域中

P type和N type都会提高电导率

没有外部输入的静态的就是热平衡状态

k是一个常数

pn diode 规定：

p-side=anode
n-side=cathode

Lecture 3

Lecture 4

只要你试出一种状态，并且所有二极管的验证条件都满足，就可以停止，不需要继续尝试其他组合。

| Diode model | 导通条件 | ON 等效 | 截止条件 | OFF 等效 |
| --- | --- | --- | --- | --- |
| Ideal diode model 理想模型 | vD=0, iD≥0 | 短路，vD=0 | vD≤0, iD=0 | 开路 |
| Constant voltage drop model 恒压降模型 | vD=VON, iD≥0 | 电压源，硅管常取 vD=0.7V | vD<VON, iD=0 | 开路 |
| Piecewise-linear model 分段线性模型 | vD≥VON, iD≥0 | vD=VON+iDrD | vD<VON, iD=0 | 开路 |
| Exponential model 指数模型 | 没有硬阈值；vD>0 时电流指数增大 | iD=ISevD/(nVT)1 | vD<0时只有很小反向电流 | 同一指数公式 |

Lecture 5

因为输出电压跟随输入电压：

vo=vs

电压增益：

Av=vovs=1

3. 如果没有 op amp，会发生什么？

假设信号源有内阻 Rs，负载是 RL。

如果直接连接，那么输出到负载上的电压是分压：

vL=vsRLRs+RL

Lecture 6

| Quantity | Half-wave rectifier | Full-wave rectifier |
| --- | --- | --- |
| Time between charging peaks Tr | T | T2 |
| Charging time / diode ON time ΔT | ΔT=1ω2VRVp | ΔT=1ω2VRVp |
| Discharging time t1 | t1=T-ΔT | t1=T2-ΔT |
| Ripple voltage VR | VR≈VM1e-(T-ΔT)/(RC) | VR≈VM1e-(T/2-ΔT)/(RC) |
| Ripple voltage approximation | VR≈VM(T-ΔT)RC≈VMTRC | VR≈VM(T/2-ΔT)RC≈VMT2RC |
| DC output voltage VDC | VDC≈VM-VR2 | VDC≈VM-VR2 |
| DC load current IDC | IDC≈VDCR | IDC≈VDCR |
| Conduction angle θCunder your notation | θC=ωΔT≈2VRVp | θC=ωΔT≈2VRVp |

Lecture 7

Lecture 8